| Urbana Mecánica // Maza Contrapedal Single Speed Mecánica // Single Speed Frenos y Palancas // Horquillas y Palancas al Manubrio | Híbrida |
| --- | --- |
| - Bottom Braket // Square Sealed Bearing 110mm - Crankset // 44 – 48 T - Free Wheel // 18 – 22 T - Chain // 96 Links - Rims Size // 700 - Tires // 700x28c - Fork Size // 700 Over 1 1/8 Straight – threadless - Headset // Over External 1 1/8 (31.8mm) - Seat Post // 27.2mm - Seat Clamp // 31.8mm Bolt - Steam // OVER – OVER 80mm - HandleBar // Riser OVER - Grips // MTB - Saddle // Bars - Peddals // 9/16 check compatibility with crankset | - Bottom Braket // Square Sealed Bearing 113mm or Hollow tech bearings - Crankset // 36D or 39-50D - Cassette // 8 – 10 steps (11/36-46) - Chain // Related to the number of steps - Transmition // 8 – 10 step Rear derailleur and Levers - Hubs // Quick Release 8 – 10 steps, Disc Brake ready, check compatibility with Rims (check hole count) - Front Derailleur – Only for double chainring, direct mount - Rims // 700 (32 o 36 holes) - Disc Brakes // 160mm Rotors (check compatibility with hubs), mechanical calippers, MTB levers, R160 + F160 adaptors - Tires // 700×32-40 - Fork // Over 1 1/8 Threadless (check compatibility with tire) - Headset // Over External 1 1/8 (31.8mm) - Seat Post // 27.2mm - Seat Clamp // 31.8mm Bolt - Steam // OVER – OVER 80mm - Handle Bar // Riser OVER - Grips // MTB - Saddle // Bars - Peddals // 9/16 (check compatibility with crankset ) |
| Wheel size 700 Tire 700×23-28 Rims 700 – 32 o 36 holes Spokes 14G Single Speed o Coasterbrake | Wheel size 700 Tires 700×32-40 Rims 29 – 32 o 36 holes Hubs 32 o 36 holes (check compatibility with Cassette) Cassette 8, 9, 10 u 11 steps (check compatibility with Hub) Disc Brake Rotor (Center Lock or 6 Knots (check compatibility with Hub) Spokes 14G |
| Fork Size 700 OVER 1 1/8 Threadless Carbon, Aluminium, Steel or Bamboo | Fork Size 700 For wheels 700 x 32-80 OVER 1 1/8 Recta (NO CONIC) Disc Brake or Vbrake Ready Carbon, Aluminium, Steel or Bamboo |